#define F_CPU 8000000UL

#include <util/delay.h>

#include <avr/io.h>

#include <avr/interrupt.h>

#include <stdio.h>

#include "LCD.h"

#define SIM800_TX_PORT      PORTD

#define SIM800_TX_PIN       PD1

#define SIM800_RX_PORT      PORTD

#define SIM800_RX_PIN       PD0

// Flag

volatile uint8_t smsFlag = 1; // Initialize flag to 1 initially

// BuadRate

unsigned int BuadRate = 9600;

// Global char

char buffer[20];

char Phone[] = "+989918726450"; // Replace with the recipient's phone number

char GasMessage[] = "AirQulity is Bad";

char NoGasMessage[] = "AirQulity is Good";

void ADC_Init(){

DDRA = 0x00; // Set PORTA as input

ADCSRA = 0x87; // Enable ADC, set prescaler to 128

ADMUX = 0x40; // AVCC with external capacitor at AREF pin

}

int ADC_Read(char channel){

int A, ALow;

ADMUX = (ADMUX & 0xF0) | (channel & 0x0F); // Select ADC channel

ADCSRA |= (1 << ADSC); // Start conversion

while(!(ADCSRA & (1 << ADIF))); // Wait for conversion to complete

_delay_us(10);

ALow = (int)ADCL;

A = (int)ADCH * 256;

A = A + ALow;

return A;

}

// SMS Sender

void GSM_SendSMS(char* phone, char* message, int Air) {

// Send AT command to set SMS mode

LCD_Clear();

LCD_String_xy(0, 1, "Set GSM");

send_AT_command("AT+CMGF=1\r");

_delay_ms(1000); // Wait for response

// Send AT command to set recipient phone number

LCD_Clear();

LCD_String_xy(0, 4, "Phone: ");

LCD_String_xy(1, 3, phone);

char command[30];

sprintf(command, "AT+CMGS=\"%s\"\r", phone);

send_AT_command(command);

_delay_ms(1000); // Wait for response

// Send the SMS message

LCD_Clear();

LCD_String_xy(0, 4, "Send SMS");

send_AT_command(message);

_delay_ms(100); // Wait for response

// Send Air Quality

LCD_Clear();

LCD_String_xy(0, 1, "Send Data");

char Data[30];

sprintf(Data, "AirQuality: %d", Air);

send_AT_command(Data);

_delay_ms(500);

// Send Ctrl+Z to indicate the end of the message

LCD_Clear();

LCD_String_xy(0, 1, "Done");

UART_Transmit(0x1A);

_delay_ms(100); // Wait for response

// Wait for SMS to be sent

_delay_ms(500);

smsFlag = 0; // Reset the flag after sending SMS

}

// USART Initialize

void UART_Init(unsigned int baud_rate) {

// Set baud rate

unsigned int ubrr = F_CPU / 16 / baud_rate - 1;

UBRRH = (unsigned char)(ubrr >> 8);

UBRRL = (unsigned char)ubrr;

// Enable receiver and transmitter

UCSRB = (1 << RXEN) | (1 << TXEN);

// Set frame format: 8 data bits, 1 stop bit, no parity

UCSRC = (1 << URSEL) | (1 << UCSZ1) | (1 << UCSZ0);

}

// Function to transmit a character via UART

void UART_Transmit(unsigned char data) {

// Wait for empty transmit buffer

while (!(UCSRA & (1 << UDRE)));

// Put data into buffer, sends the data

UDR = data;

}

// Function to transmit an AT command

void send_AT_command(const char *command) {

// Transmit each character of the command

while (*command) {

UART_Transmit(*command);

command++;

}

// Send carriage return and line feed

UART_Transmit('\r');

UART_Transmit('\n');

}

// Timer1 Initialize

void Timer_init() {

// Set Timer1 in normal mode with scale 1024

TCCR1A = 0;

TCCR1B = (1 << CS12) | (1 << CS10);

// Enable Timer1 overflow interrupt

TIMSK |= (1 << TOIE1);

// Set Timer1 value for a 1-minute interval

TCNT1 = 65535 - (F_CPU / 61440); // Adjust based on your MCU frequency

// take 7.5 sec to overflow Timer1

}

// Interrupt

ISR(TIMER1_OVF_vect){

static uint16_t timer_count = 0;

timer_count++;

if(timer_count >= 8) { // 8 * 7.5 = 1 minute

smsFlag = 1;

timer_count = 0;

}

}

int main(void)

{

// Set USART

UCSRC = 0x86; //0b10000110

// UCSZ1 & UCSZ0 == we use 8-bit

UBRRH = 0x00; //0b00000000

UBRRL = 0x06; //0b00000110

DDRA = 0x00; // Set PORTA as input

DDRD = 0b11111100; // Set PORTD as output except PD0 and PD1

UART_Init(BuadRate);

Timer_init();

ADC_Init();

LCD_Init();

sei();

int AirValue;

int AirSensor;

int Pot_Value;

// Welcome

LCD_Clear();

LCD_String_xy(0, 4, "Welcome");

_delay_ms(1000);

while (1)

{

AirValue = ADC_Read(0);

Pot_Value = ADC_Read(1);

float AirBuffer;

_delay_ms(100);

if (Pot_Value < 128 )

{

AirBuffer = AirValue * 0.125;

}

else if (Pot_Value < 256)

{

AirBuffer = AirValue * 2 /8 ;

}

else if (Pot_Value < 384)

{

AirBuffer = AirValue * 3 / 8 ;

}

else if (Pot_Value < 512)

{

AirBuffer = AirValue *  4 / 8 ;

}

else if (Pot_Value < 640)

{

AirBuffer = AirValue * 5 / 8 ;

}

else if (Pot_Value < 768)

{

AirBuffer = AirValue * 6 / 8 ;

}

else if (Pot_Value < 896)

{

AirBuffer = AirValue * 7 / 8 ;

}

else if (Pot_Value < 1024)

{

AirBuffer = AirValue * 8 / 8 ;

}

_delay_ms(100);

//AirBuffer = 500.55;

AirSensor = (int)AirBuffer;

// Show AirQuality

if (AirSensor > 580)

{

LCD_Clear();

sprintf(buffer, "AirQuality: %d", AirSensor);

LCD_String_xy(0, 1, buffer);

LCD_String_xy(1, 4, "Gas");

PORTD |= (1 << 2);

PORTD |= (1 << 3);

PORTD |= (1 << 4);

}

else

{

LCD_Clear();

sprintf(buffer, "AirQuality: %d", AirSensor);

LCD_String_xy(0, 1, buffer);

LCD_String_xy(1, 4, "NoGas");

PORTD &= ~(1 << 2);

PORTD &= ~(1 << 3);

PORTD &= ~(1 << 4);

}

// Send SMS

if (smsFlag == 1)

{

_delay_us(50);

if (AirSensor > 580)

{

GSM_SendSMS(Phone, GasMessage, AirSensor);

}

else

{

GSM_SendSMS(Phone, NoGasMessage, AirSensor);

}

smsFlag = 0; // Reset the flag after sending SMS

}

_delay_ms(250);

LCD_Clear();

}

}